# FORD AUTOMOTIVE

# LOCKSMITH PROGRAMMING GUIDE

Comprehensive Technical Reference for Professional Locksmiths

North American Market Vehicles

Covering PATS Systems, Transponder Programming,

Smart Key Systems, and Diagnostic Procedures

CONFIDENTIAL - FOR PROFESSIONAL USE ONLY

# TABLE OF CONTENTS

1. Ford PATS System Overview

2. Transponder Chip Evolution and Specifications

3. Vehicle-Specific Key Programming Data

3.1 F-150 / F-Series Super Duty

3.2 Explorer

3.3 Mustang

3.4 Edge / Fusion

3.5 Escape

3.6 Expedition

3.7 Bronco / Bronco Sport

3.8 Ranger / Maverick

3.9 Focus / Fiesta

3.10 Transit / Transit Connect

3.11 Flex / Taurus / EcoSport

4. On-Board Programming Procedures

5. Autel IM608 Programming Procedures

6. All Keys Lost (AKL) Procedures

7. Lishi Tool Compatibility

8. Critical Warnings and Compatibility Notes

9. Quick Reference Tables

# 1. FORD PATS SYSTEM OVERVIEW

The Passive Anti-Theft System (PATS) is Ford's immobilizer technology that has been standard on Ford, Lincoln, and Mercury vehicles since 1996. Understanding the different PATS generations is critical for successful key programming.

## 1.1 PATS Generations

### PATS I (1996-1998)

- Only ONE working key required for programming

- Standalone PATS control module

- Maximum of 16 keys can be programmed to vehicle

- Simpler security protocol

### PATS II (1998-2000+)

- TWO working keys required for on-board programming

- Anti-scan function: 30-second lockout after failed programming attempt

- Most common system found in Ford vehicles

- Maximum of 8 keys can be programmed

### E-PATS / Enhanced PATS (2000+)

- Encrypted transponder communication

- Enhanced security features with rolling codes

- Integrated with PCM or BCM depending on model year

## 1.2 PATS Control Function Types

Ford uses different PATS control function types (A through G) that determine how key data is stored and what procedures are required after module replacement:

| Type | Description and Requirements |
| --- | --- |
| A/D | Key data NOT stored in PCM. No key relearn required after PCM replacement. Keys stored in separate PATS module. |
| B/C | Key data stored in PCM. Must transfer key data or perform key relearn after PCM replacement. |
| E | Key data stored in PCM. CANNOT transfer - must perform key relearn on vehicle after replacement. |
| F/G | Newer implementations with key data in BCM. Requires parameter reset and key programming via scan tool. |

## 1.3 Parameter Reset vs Key Relearn

Understanding the difference between Parameter Reset and Key Relearn is critical for PCM/BCM replacement scenarios:

Parameter Reset: Required for Type B/C/F/G when only the PCM is replaced. Requires scan tool or J2534 device with Ford Module Programming (FMP) subscription. A 10-minute security access period is required before parameter reset can be performed.

Key Relearn: Required when PCM is replaced and key data cannot be transferred from the old module. All keys must be re-programmed to the new module.

# 2. TRANSPONDER CHIP EVOLUTION

Ford has utilized several transponder chip technologies throughout its PATS evolution. Correct chip identification is essential for successful key programming.

## 2.1 Chip Type Reference Chart

| Chip ID | Technology | Years Used | Notes |
| --- | --- | --- | --- |
| ID 4C | Texas Instruments Fixed Code | 1996-1998 | Early PATS I systems |
| ID 4D60 | Texas Crypto 40-bit | 1998-2005 | PATS II introduction |
| ID 4D63 (40-bit) | Texas Crypto 40-bit | 2000-2010 | Upgradeable when programmed |
| ID 4D63 (80-bit) | Texas Crypto 2 / DST80 | 2005-2017 | Most common Ford chip |
| ID 83 | Texas 4D63 80-bit variant | 2012-2019 | Focus, C-Max, Escape Titanium |
| ID 49 / HITAG Pro | NXP 128-bit AES | 2015-Present | NCF2951F/PCF7939FA chips |

## 2.2 Chip Designation Notes

- ID 4D63 chips are available in both 40-bit and 80-bit versions. The 80-bit version can be "downgraded" to work with 40-bit systems but not vice versa.

- ID 83 is essentially a variant designation for the 80-bit 4D63 chip used in specific Ford models - some programming tools label these differently.

- HITAG Pro (ID49) uses NXP NCF2951F or PCF7939FA/PCF7953P chips with 128-bit AES encryption.

- When ordering keys, always verify both the chip type AND frequency to ensure compatibility.

# 3. VEHICLE-SPECIFIC KEY PROGRAMMING DATA

## 3.1 Ford F-150 / F-Series Super Duty

### F-150 2015-2017 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A2C31243300 |
| Frequency | 902 MHz |
| Chip | HITAG Pro ID49 128-bit (NCF2951F/PCF7953P) |
| Part Number | 164-R8117 |
| Battery | CR2025 x2 |
| Emergency Blade | HU101 |
| Buttons | 5 (Lock, Unlock, Panic, Remote Start, Tailgate) |

### F-150 2018-2024 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A2C93142600 / M3N-A2C931426 |
| Frequency | 902 MHz |
| Chip | HITAG Pro ID49 128-bit |
| Part Number | 164-R8166, 5929503, 5933019 |
| Battery | CR2450 |
| Emergency Blade | HU101 |
| Buttons | 5 (Lock, Unlock, Panic, Remote Start, Tailgate) |

⚠️ WARNING: 2024 F-150 models cannot self-program - requires locksmith/dealer tools.

### F-150 2015-2024 Flip Key

| Specification | Details |
| --- | --- |
| FCC ID | N5F-A08TAA |
| Frequency | 315 MHz |
| Chip | HITAG Pro ID49 128-bit |
| Part Number | 164-R8130, 164-R8269 |
| Battery | CR2032 |
| Keyway | HU101 |
| Buttons | 3 (Lock, Unlock, Trunk/Tailgate) |

### F-250/F-350/F-450/F-550 Super Duty 2017-2024

Super Duty models share specifications with F-150 2018+ smart keys:

FCC ID: M3N-A2C93142600 / M3N-A2C931426

Frequency: 902 MHz

Chip: HITAG Pro ID49 128-bit

Part Numbers: 164-R8166, 164-R8198

## 3.2 Ford Explorer

### Explorer 2011-2015 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N5WY8609 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit |
| Part Number | 164-R8091 |
| Battery | CR2032 |
| Buttons | 4 (Lock, Unlock, Panic, Trunk) |

### Explorer 2016-2017 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A2C31243300 (4-button variant) |
| Frequency | 902 MHz |
| Chip | HITAG Pro ID49 128-bit |
| Part Number | 164-R8140, GB5T-15K601-CA |
| Battery | CR2025 x2 |

### Explorer 2018-2022 Smart Key

Multiple configurations available - verify FCC ID before ordering:

4-Button (315 MHz): FCC ID: M3N-A2C93142300, Part #: 164-R8150

5-Button (902 MHz): FCC ID: M3N-A2C931426, Part #: 164-R8198

Battery: CR2450

Chip: HITAG Pro ID49 128-bit

### Explorer 2024 Smart Key

⚠️ IMPORTANT: 2024 Explorer uses a different frequency!

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A3C108397 |
| Frequency | 434 MHz (NEW FREQUENCY) |
| Chip | HITAG Pro ID49 128-bit |
| Part Number | 164-R8397, PL1T-15K601-BB |
| Battery | CR2450 |
| Features | 5-button with motion sensing |

## 3.3 Ford Mustang

### Mustang 2011-2014 Remote Head Key

| Specification | Details |
| --- | --- |
| FCC ID | CWTWB1U793 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit |
| Part Number | 164-R8087 |
| Buttons | 4 (Lock, Unlock, Trunk, Panic) |

### Mustang 2015-2017 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A2C31243300 |
| Frequency | 902 MHz |
| Chip | HITAG Pro ID49 128-bit |
| Part Number | 164-R8119 |
| Battery | CR2025 x2 |
| Buttons | 5 |

### Mustang 2018-2022 Smart Key

Multiple configurations - verify frequency before ordering:

4-Button (315 MHz): FCC ID: M3N-A2C93142300, Part #: 164-R8150, 164-R8159

5-Button (902 MHz): FCC ID: M3N-A2C931426 / M3N-A2C93142600, Part #: 164-R8162

Battery: CR2450

Chip: HITAG Pro ID49 128-bit

Note: 2017-2018 transition years may have FCC ID compatibility despite different ID numbers - verify frequency match.

## 3.4 Ford Edge / Fusion

### Edge 2015-2016 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A2C31243800 |
| Frequency | 315 MHz |
| Chip | HITAG Pro ID49 |
| Part Number | 164-R8109 |
| Battery | CR2032 |
| Buttons | 4 (Lock, Unlock, Trunk, Panic) |

### Edge / Fusion 2017-2022 Smart Key

Available in multiple frequency configurations:

315 MHz Version: FCC ID: M3N-A2C93142300, Part #: 164-R8150 (4-button)

902 MHz Version: FCC ID: M3N-A2C93142600, Part #: 164-R8149 (5-button)

Chip: HITAG Pro ID49 128-bit

Battery: CR2450

### Fusion 2013-2016 Flip Key

| Specification | Details |
| --- | --- |
| FCC ID | N5F-A08TAA |
| Frequency | 315 MHz |
| Chip | HITAG Pro ID49 |
| Part Number | 164-R7986, 5924003 |
| Buttons | 4 |

## 3.5 Ford Escape

### Escape 2013-2019 Smart Key (Titanium Trim)

| Specification | Details |
| --- | --- |
| FCC ID | M3N5WY8609 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit (ID83) |
| Part Number | 164-R7995, CJ5T-15K601-Dx |
| Battery | CR2032 |
| Buttons | 5 |

### Escape 2013-2019 Remote Head Key (Non-Titanium)

| Specification | Details |
| --- | --- |
| FCC ID | OUCD6000022 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit |
| Part Number | 164-R8046, 5921709 |
| Keyway | HU101 |
| Buttons | 4 |

### Escape 2020-2024 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N-A2C931426 |
| Frequency | 902 MHz |
| Chip | HITAG Pro ID49 128-bit |
| Part Number | 164-R8198 |
| Battery | CR2450 |

## 3.6 Ford Expedition

### Expedition 2018-2022 Smart Key

Available in multiple button configurations:

5-Button (902 MHz): FCC ID: M3N-A2C931426, Part #: 164-R8198

3-Button (434 MHz): FCC ID: M3N-A2C93142100, Part #: 164-R8151

Chip: HITAG Pro ID49 128-bit (NCF2951F)

Battery: CR2450

### Expedition 2018-2022 Flip Key

| Specification | Details |
| --- | --- |
| FCC ID | N5F-A08TAA |
| Frequency | 315 MHz |
| Chip | HITAG Pro ID49 |

## 3.7 Ford Bronco / Bronco Sport

### Bronco 2021-2024 Smart Key

Multiple configurations available:

4-Button (315 MHz): FCC ID: M3N-A2C931423, Part #: 164-R8287, Chip: HITAG Pro ID49

4-Button (902 MHz): FCC ID: M3N-A2C931426, Part #: 164-R8182, 164-R8297

5-Button (902 MHz): FCC ID: M3N-A2C931426, Part #: 164-R8288

Bronco Raptor: Part #: 164-R8340

Battery: CR2450

3-Button (315 MHz): FCC ID: M3N-A2C931423, Part #: 164-R8295, 5940319

### Bronco Sport 2021-2024

Uses same key configurations as standard Bronco. Verify frequency (315 MHz vs 902 MHz) matches original before programming.

## 3.8 Ford Ranger / Maverick

### Ranger 2019-2024 Flip Key

| Specification | Details |
| --- | --- |
| FCC ID | N5F-A08TAA |
| Frequency | 315 MHz |
| Chip | HITAG Pro ID49 |

### Maverick 2022-2024

Flip Key: FCC ID: N5F-A08TAA, 315 MHz

Smart Key: FCC ID: M3N5WY8609, 315 MHz, ID83 chip

## 3.9 Ford Focus / Fiesta

### Focus 2012-2018 Remote Head Key

| Specification | Details |
| --- | --- |
| FCC ID | OUCD6000022 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit |
| Part Number | 164-R8046 |
| Keyway | HU101 |

### Focus 2012-2017 Smart Key (SE/Titanium)

| Specification | Details |
| --- | --- |
| FCC ID | M3N5WY8609 |
| Frequency | 315 MHz |
| Chip | ID83 (Texas 4D63 80-bit variant) |
| Part Number | 164-R7995 |

### Fiesta 2011-2019

Flip Key: FCC ID: N5F-A08TAA, 315 MHz, HITAG Pro ID49

Smart Key (2012+): FCC ID: M3N5WY8609, 315 MHz

Remote Head Key: FCC ID: KR55WK47899, 315/433 MHz, ID 4D60/4D63

## 3.10 Ford Transit / Transit Connect

### Transit Connect 2014-2020 Remote Head Key

| Specification | Details |
| --- | --- |
| FCC ID | OUCD6000022 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit (some models non-transponder) |
| Part Numbers | 164-R8046 (with chip), 164-R8007 (3-button) |
| Keyway | HU101 |

Note: Some Transit Connect models do not use a transponder chip. Verify original key configuration before ordering.

### Transit 2019-2024 Smart Key

FCC ID: M3N-A2C931423

Frequency: 315 MHz

Chip: HITAG Pro ID49

## 3.11 Ford Flex / Taurus / EcoSport

### Flex 2012-2019 Smart Key

| Specification | Details |
| --- | --- |
| FCC ID | M3N5WY8609 |
| Frequency | 315 MHz |
| Chip | Texas ID 4D63 80-bit |
| Part Number | 164-R8091 |
| Buttons | 4 |

### Taurus 2014-2019 Smart Key

FCC ID: M3N5WY8609

Frequency: 315 MHz

Chip: Texas ID 4D63 80-bit

Part Number: 164-R8091

### EcoSport 2018-2022

Smart Key (315 MHz): FCC ID: M3N-A2C93142300, Part #: 164-R8163

Chip: HITAG Pro ID49

# 4. ON-BOARD PROGRAMMING PROCEDURES

On-board programming (OBP) allows key programming without specialized equipment when the customer has two working keys. These procedures vary by vehicle type.

## 4.1 Two-Key Learning Procedure (PATS II - Keyed Ignition)

Standard procedure for vehicles with keyed ignition and PATS II system. Requires TWO previously programmed keys.

- Insert first programmed PATS key into ignition

- Turn ignition to RUN position (not START)

- Wait for security/theft indicator light to go out (approximately 3-5 seconds)

- Turn ignition to OFF position and remove key

- Within 10 seconds, insert second programmed PATS key

- Turn ignition to RUN position

- Wait for security indicator to go out

- Turn ignition to OFF and remove key

- Within 10 seconds, insert NEW unprogrammed key

- Turn ignition to RUN position

- Wait for security indicator to go out (confirms programming)

- Turn OFF and wait 30 seconds before testing

- Repeat steps 9-12 for additional keys

## 4.2 Push-Button Start Smart Key Programming

For vehicles with push-button start and intelligent access system. Requires TWO previously programmed smart keys.

- Place first programmed smart key in backup slot (center console, cupholder, or glove box)

- Press START/STOP button without pressing brake (accessory mode)

- Wait 5 seconds

- Press START/STOP button again

- Remove first key from backup slot

- Wait 5 seconds, then place second programmed key in backup slot

- Press START/STOP button

- Wait 5 seconds, press START/STOP button again

- Remove second key from backup slot

- Wait 5 seconds, place NEW unprogrammed key in backup slot

- Press START/STOP button and HOLD until door locks cycle

- Door lock cycle confirms successful programming

- Test all remotes for proper operation

## 4.3 Remote Keyless Entry (RKE) Programming

For programming the remote portion of keys (lock/unlock buttons). This is separate from transponder programming.

- Close all doors, then open driver's door

- Press unlock button on driver's door panel

- Insert key in ignition

- Turn ignition from OFF to RUN 8 times within 10 seconds, ending in RUN

- Door locks will cycle to confirm programming mode

- Within 20 seconds, press any button on first remote

- Door locks will cycle to confirm remote accepted

- Repeat for additional remotes (up to 8 total)

- Turn ignition to OFF to exit programming mode

## 4.4 Backup Slot Locations by Model

| Model | Backup Slot Location |
| --- | --- |
| F-150 (2015-2024) | Center console cupholder area |
| Explorer (2011-2024) | Center console armrest box |
| Mustang (2015-2022) | Center console cupholder |
| Edge (2015-2022) | Center console cupholder |
| Escape (2013-2024) | Center console armrest |
| Expedition (2018-2022) | Center console cupholder |
| Bronco (2021-2024) | Center console |

# 5. AUTEL IM608 PROGRAMMING PROCEDURES

## 5.1 Standard Menu Navigation

IMMO → Ford → Manual Selection → Ford → USA → [Model] → [Year Range] → Smart Key/Transponder Key → Control Unit → Key Learning

## 5.2 Add Key Procedure (With Working Keys)

- Requires 2 working keys for most PATS II systems

- Place keys in backup slot when prompted

- Follow on-screen prompts for key insertion sequence

- System may prompt for ignition cycles

## 5.3 All Keys Lost (AKL) Procedure

Menu Path: Key Learning → All Smart Keys Lost

- Network connection required (pay-per-use function for some models)

- Must program minimum 2 keys to complete AKL

- System will delete all previously learned keys

## 5.4 2021+ Ford RH850 BCM Procedure

For 2021+ F-150, Bronco, Mustang Mach-E, and similar vehicles with RH850 processor BCM:

- Locate BCM - passenger side footwell, under instrument panel

- Remove BCM from vehicle

- Connect BCM to XP400 Pro with APB131 adapter

- Navigate: Ford → F-150 → 2021-2023 → Smart Key → Control Unit → Keyless System (CAN) → Key Learning → Read Dflash Data

- Read and save BCM D-flash data

- Reinstall BCM in vehicle

- Program keys via OBD with loaded D-flash data

## 5.5 NASTF/VSP Requirement Notice

⚠️ CRITICAL: As of recent updates, Autel IM508/IM608 may require NASTF (National Automotive Service Task Force) login for 2015+ Ford key programming. Some functions may be restricted without VSP (Vehicle Security Professional) credentials. Verify current tool capabilities before purchase.

## 5.6 XP400 Pro Requirements

- Key generation for HITAG Pro ID49 chips

- D-flash reading for 2021+ models (requires APB131 adapter)

- EEPROM backup/restore functions

- Bench programming when OBD fails

# 6. ALL KEYS LOST (AKL) PROCEDURES

All Keys Lost scenarios require specialized equipment and procedures. The approach varies significantly by model year and PATS generation.

## 6.1 Pre-2015 Vehicles

- Older PATS systems may allow incode/outcode calculation

- Some require 10-minute security timer

- Programming via scan tool with appropriate security access

## 6.2 2015-2020 Vehicles

- Requires scan tool with Ford AKL capability

- Network connection typically required for server authentication

- Pay-per-use tokens may be required

- NASTF VSP credentials may be necessary

## 6.3 2021+ Vehicles (RH850 BCM)

Newest Ford vehicles use RH850 processor-based BCM requiring bench work:

- BCM must be physically removed for D-flash extraction

- Requires XP400 Pro with APB131 adapter (Autel) or equivalent

- Alternative tools: OBDSTAR G3 with RH850 adapter, Abrites with ZN085

- Some BCM versions (MU5T) may only be programmable via Ford FDRS

- Supported BCM: R7F701573 version

## 6.4 Ford FDRS/IDS Alternative

For vehicles not supported by aftermarket tools, Ford's dealer-level diagnostic system remains an option:

- Requires J2534 passthru device (VCI200, VXDIAG, etc.)

- Ford subscription required

- Most reliable method for newest vehicles

- Supports all PATS functions including parameter reset

# 7. LISHI TOOL COMPATIBILITY

## 7.1 FO38 (H75) 2-in-1 Pick/Decoder

| Specification | Details |
| --- | --- |
| Profile | 8-cut |
| Capabilities | Ignition, door, trunk |
| Years | 1997-2011 |
| Keyway | FO38/H75 |

Compatible Vehicles: Crown Victoria, Escape (2001-2011), Expedition (1997-2011), Explorer (1997-2010), F-150/F-250/F-350 (1997-2011), Mustang (1997-2011), Ranger (1997-2011), Taurus (1997-2009), Lincoln Town Car, Mercury Grand Marquis, Mariner

## 7.2 HU101 V3 2-in-1 Pick/Decoder

| Specification | Details |
| --- | --- |
| Profile | 10-cut high-security |
| Design | Cutout version, double lifter |
| Capabilities | Ignition, door, trunk |
| Years | 2011-2024+ |
| Keyway | HU101 |

Compatible Vehicles: C-Max, Edge (2011+), Escape (2013+), Fiesta (2011+), Focus (2012+), Fusion (2013+), Mustang (2015+), Explorer (2011+), F-150 (2015+), F-250/F-350/F-450 (2017+), Transit, Transit Connect

Also fits: Jaguar, Land Rover, Lincoln, Volvo (shared HU101 platform)

## 7.3 FO38 Key Reader

- Decodes door, trunk, ignition locks without picking

- Reads bitting directly from lock

- Faster than pick/decode for key origination

- Compatible with same vehicles as FO38 2-in-1

# 8. CRITICAL WARNINGS AND COMPATIBILITY NOTES

## 8.1 Frequency Compatibility

⚠️ CRITICAL: Keys may appear physically identical but operate on different frequencies. ALWAYS verify FCC ID before ordering!

| Frequency | Usage Notes |
| --- | --- |
| 315 MHz | Older systems, flip keys, some 2017-2022 smart keys |
| 434 MHz | 2024+ Explorer, some export models |
| 902 MHz | Newer smart key systems, 2015+ premium models |

## 8.2 Theft Indicator Light Diagnostics

| Light Behavior | Meaning |
| --- | --- |
| Solid 2 sec then OFF | Normal - key recognized |
| Rapid flashing | Key not recognized or anti-scan active (wait 30 seconds) |
| Continuous flashing | PATS system fault |
| No illumination | System malfunction - check PATS module/transceiver |

## 8.3 Common Pitfalls

- 40-bit vs 80-bit chips: Ensure correct chip bit rate. 80-bit keys will NOT work in 40-bit systems.

- ID63 vs ID83 labeling: Same chip, different tool designations. Verify compatibility.

- 2024 F-150: Cannot self-program. Requires professional locksmith/dealer tools.

- Explorer 2024: Frequency changed to 434 MHz from previous 315/902 MHz.

- Aftermarket keys: Some aftermarket keys ship with incorrect chips. Verify before programming.

- Transit Connect: Some models have NO transponder chip. Verify original configuration.

## 8.4 Programming Timeouts

- All on-board programming steps must be completed within specified time windows (typically 10-30 seconds)

- If anti-scan activates (rapid flashing), wait full 30 seconds before retrying

- After failed attempt, wait 60 seconds before restarting entire procedure

- Battery voltage should be above 12.2V for reliable programming

# 9. QUICK REFERENCE TABLES

## 9.1 Emergency Blade Keyways

| Keyway | Profile | Years |
| --- | --- | --- |
| FO38/H75 | 8-cut | 1997-2011 |
| HU101 | 10-cut high-security (2-track) | 2011-2024 |

## 9.2 Battery Reference

| Battery | Typical Applications |
| --- | --- |
| CR2032 | Most 315 MHz remotes, flip keys, older smart keys |
| CR2025 x2 | Some 902 MHz smart keys (2015-2017 models) |
| CR2450 | Most 2017+ smart keys |

## 9.3 Common FCC ID Quick Reference

| FCC ID | Frequency | Common Models |
| --- | --- | --- |
| M3N5WY8609 | 315 MHz | Explorer 11-15, Flex, Taurus, Escape Titanium |
| M3N-A2C31243300 | 902 MHz | F-150 15-17, Explorer 16-17, Mustang 15-17 |
| M3N-A2C93142300 | 315 MHz | Edge, Fusion, Explorer, Mustang 17-22 (4B) |
| M3N-A2C93142600 | 902 MHz | F-150 18+, Edge, Fusion, Mustang 17-22 (5B) |
| M3N-A2C931426 | 902 MHz | Expedition, Escape 20+, Bronco, Explorer 18+ |
| N5F-A08TAA | 315 MHz | Universal flip key - F-150, Ranger, Fusion, etc. |
| OUCD6000022 | 315 MHz | Focus, Escape, C-Max, Transit Connect (RHK) |
| CWTWB1U793 | 315 MHz | Mustang 11-14, older Explorer, F-150 (RHK) |

END OF DOCUMENT

This guide is intended for professional locksmith use only.

Always verify specifications against original key before ordering or programming.

Document Version 1.0 - December 2024