make,model,year_start,year_end,module_name,module_location,main_chip,eeprom_chip,pin_via_obd,akl_method,tools_supported,notes
Chevrolet,Silverado,2014,2019,BCM,Under dash left of steering column,Freescale MC9S12XEQ384,95320 SPI EEPROM[1],No,Read BCM EEPROM (via OBD adapter or on bench),Autel IM608 / VVDI2,Immobilizer data stored in BCM flash/EEPROM
Ford,F-150,2015,2020,PCM (PATS module),Behind glove box (integrated),N/A (As-built data),N/A,Yes,OBD PATS reset (no disassembly),Autel IM608 / Smart Pro,Requires 10-minute security wait or dealer code (J2534 or incode/outcode)
Honda,Accord,2018,2024,Immobilizer (IMMO),Behind gauge cluster,Renesas RH850 MCU,93C66 EEPROM,No,Remove cluster & read IMMO EEPROM,K518 / OBDSTAR X300,No PIN available – must dump IMMO data to program new key
Toyota,Camry,2018,2024,Smart ECU,Center console (behind dash),N/A (Smart key ECU),N/A,Yes,OBD programming with smart key emulator[2],Autel IM608 / Xhorse VVDI,Requires special emulator (APB112) to bypass 16-min wait for AKL
Hyundai,Sonata,2015,2019,BCM,Driver kick panel,ST Micro ST10F MCU,24C04 EEPROM,No,Direct EEPROM read from BCM,Tango / VVDI Prog,8-pin SOIC clip needed to read immobilizer eeprom data
Nissan,Altima,2013,2018,BCM (Intelligent Key),Behind passenger glove box,NEC/Renesas MCU (V850 series),93C66 or 95080 EEPROM[3],No,Read BCM eeprom dump or obtain 20-digit PIN,Autel IM608 / Smart Pro,12- or 20-digit PIN code required for programming (calc from BCM dump or Nissan server)
Jeep,Grand Cherokee,2005,2010,SKIM (Immobilizer Module),Steering column near ignition cylinder,Motorola MCU (68HC08),24C16 EEPROM[4],No,Remove SKIM and read EEPROM dump,Orange5 / UPA-USB / Autel,4-digit PIN must be pulled from EEPROM dump (no OBD read; keys then programmed via scan tool)

Sources:

- Digicar Expert EEPROM list confirming Chevrolet Silverado (2014–2019) BCM contains a 95320 EEPROM for immobilizer data[1].

- Autel documentation on APB112 emulator showing Toyota all-keys-lost can be done via OBD with a smart key emulator instead of EEPROM removal[2].

- Scribd “Immo Location” guide indicating many Nissan BCMs (e.g. Sunny/Versa) use serial EEPROM (93C66/95080) to store key data[3].

- JustAnswer excerpt (2007 Jeep Grand Cherokee) noting the Jeep’s SKIM module uses an 8-pin EEPROM (type “33399”, equivalent to 24C16) which must be read to retrieve the PIN code (OBD cannot extract it)[4].

[1] Files lists

https://digicar-expert.com/en/files-lists.2

[2] Buy: Autel APB112 Smart Key Simulator - Shop Now – Autel.com

https://store.autel.com/products/autel-apb112?srsltid=AfmBOoolywR9bEAlUgFujWvZy0Sh8FG0UTIN5Qu7kWqU4Kj4uLM_BOE8

[3] Immo Location | PDF | Off Road Vehicles | Car Manufacturers

https://www.scribd.com/document/726142371/IMMO-LOCATION

[4] Jeep Grand Cherokee Key Programming Pin Code Guide

https://www.justanswer.com/chrysler/c20i0-jeep-grand-cherokee-need-pin-code-key-lost.html